dsmcFoam+ on Windows

Comprehensive Setup and Workflow Guide

OpenFOAM v1706 / hyStrath / Axisymmetric DSMC Plume Simulation

Gabriel Johnson

University of Vermont — Department of Mechanical Engineering

April 2026

# Table of Contents

1. Prerequisites and Overview

2. Installing Docker and the hyStrath Container

3. Setting Up a Case Directory

4. Importing an ANSYS Mesh into OpenFOAM

5. Converting to an Axisymmetric Wedge Mesh

6. Configuring dsmcFoam+ Case Files

7. Running dsmcFoam+

8. Post-Processing and Visualization

9. Running on the UVM VACC Cluster

Appendix A: Complete File Listing

Appendix B: Troubleshooting

# 1. Prerequisites and Overview

This guide walks through the complete process of running a DSMC plume simulation using dsmcFoam+ (part of the hyStrath framework) on a Windows machine. The workflow uses Docker to run OpenFOAM v1706 in a Linux container, avoiding the need for a native Linux installation or WSL compilation.

## 1.1 Required Software

- Docker Desktop for Windows — Provides the Linux container environment. Download from docker.com.

- Windows Terminal — Pre-installed on Windows 11. Recommended over PowerShell or CMD for interacting with Docker containers.

- ParaView — For post-processing and visualization. Download from paraview.org.

- Python 3 — For mesh conversion scripts. Comes with most systems; install from python.org if needed.

- A text editor — VS Code, Notepad++, or similar for editing OpenFOAM dictionary files.

## 1.2 Workflow Overview

The complete workflow consists of these major steps: (1) Pull and run the hyStrath Docker image. (2) Export a 2D mesh from ANSYS as a Fluent .msh file. (3) Import into OpenFOAM and convert to an axisymmetric wedge mesh. (4) Configure the dsmcFoam+ case files (species, boundaries, initial conditions). (5) Initialize particles and run the solver. (6) Post-process in ParaView.

# 2. Installing Docker and the hyStrath Container

## 2.1 Install Docker Desktop

- Download Docker Desktop from https://www.docker.com/products/docker-desktop/ and run the installer.

- During installation, ensure "Use WSL 2 instead of Hyper-V" is checked.

- Restart your computer when prompted.

- Open Docker Desktop and verify it is running (whale icon in the system tray).

- Open Windows Terminal and verify Docker works:

docker --version

docker run hello-world

## 2.2 Pull the hyStrath Docker Image

The hyStrath framework (which includes dsmcFoam+) is available as a Docker image. Pull it:

docker pull hystrath/hystrath-1706:latest

Note: If a custom image is being used (e.g., one with additional compiled utilities), use the appropriate image name. For the Benchmark Space Systems plume case, the image dsmc_v3.sif (Apptainer) or equivalent Docker image was used.

## 2.3 Start a Container

Create a working directory on your Windows machine for your OpenFOAM cases, then start the container with that directory mounted:

mkdir C:\OpenFOAM_Cases

docker run -it --name dsmc_work -v C:\OpenFOAM_Cases:/home/openfoam/cases hystrath/hystrath-1706:latest

This gives you a Linux terminal inside the container with your Windows cases directory accessible at /home/openfoam/cases. To reconnect to the container later:

docker start dsmc_work

docker exec -it dsmc_work bash

## 2.4 Verify the Installation

Inside the container, verify that dsmcFoam+ is available:

which dsmcFoam+

which dsmcInitialise+

If these commands return paths, the installation is working. If not, check that the OpenFOAM environment is sourced:

source /opt/OpenFOAM/setImage_v1706.sh

# 3. Setting Up a Case Directory

A dsmcFoam+ case follows the standard OpenFOAM directory structure:

myCase/

constant/

polyMesh/        (mesh files)

dsmcProperties   (species and collision model)

dynamicMeshDict  (adaptive mesh refinement settings)

system/

controlDict      (time stepping and output control)

boundariesDict   (DSMC boundary conditions)

dsmcInitialiseDict  (initial particle distribution)

fieldPropertiesDict (macroscopic field sampling)

controllersDict  (flux/state controllers, usually empty)

fvSchemes        (all set to 'none' for DSMC)

fvSolution       (empty for DSMC)

0/                 (created by dsmcInitialise+)

Create this structure inside your mounted cases directory:

cd /home/openfoam/cases

mkdir -p myPlumeCase/constant myPlumeCase/system

cd myPlumeCase

# 4. Importing an ANSYS Mesh into OpenFOAM

## 4.1 Exporting from ANSYS

In ANSYS Meshing (or Fluent Meshing), export your 2D mesh as a Fluent .msh file. Ensure the following before exporting:

- The mesh lies in the X-Y plane with the axis of symmetry along the X-axis (Y = 0 is the symmetry axis).

- Named selections / boundary zones are defined for inlet, outlet, wall, and axis boundaries.

- The mesh is saved in ASCII format (not binary).

## 4.2 Converting to OpenFOAM Format

Copy the .msh file into your case directory and run the Fluent mesh converter:

cp /path/to/mesh.msh .

fluentMeshToFoam mesh.msh

This creates a constant/polyMesh/ directory containing the OpenFOAM mesh files (points, faces, owner, neighbour, boundary). The result is a 3D slab mesh — your 2D mesh extruded one cell thick in the Z direction.

## 4.3 Verify the Imported Mesh

checkMesh

Check the output for: bounding box (confirms mesh extents and orientation), number of cells, geometric directions (should show 2 non-empty directions for a 2D slab), and that there are no fatal errors. Also inspect constant/polyMesh/boundary to see the patch names and types that were imported.

# 5. Converting to an Axisymmetric Wedge Mesh

This is the most involved step. The goal is to transform the flat 2D slab mesh into a thin wedge (5° total, ±2.5°) suitable for axisymmetric DSMC simulation. The process involves three sub-steps: splitting the front/back patch, rotating the points, and fixing boundary types.

Warning: The utilities makeAxialMesh and extrudeMesh may not work reliably for this conversion in hyStrath/v1706. The manual approach described here (topoSet + createPatch + Python point rotation) is the most reliable method.

## 5.1 Split the Front and Back Faces

After fluentMeshToFoam, the front and back faces of the slab are typically combined in a single patch called "frontAndBackPlanes" (or similar). These need to be split into two separate patches based on face normal direction.

### Create system/topoSetDict

This dictionary defines two face sets by selecting faces whose normals point in the +Z and -Z directions:

FoamFile

{

version     2.0;

format      ascii;

class       dictionary;

object      topoSetDict;

}

actions

(

{

name    frontFaces;

type    faceSet;

action  new;

source  normalToFace;

sourceInfo

{

normal  (0 0 1);

cos     0.5;

}

}

{

name    backFaces;

type    faceSet;

action  new;

source  normalToFace;

sourceInfo

{

normal  (0 0 -1);

cos     0.5;

}

}

);

Run topoSet:

topoSet

### Create system/createPatchDict

This dictionary creates two new patches from the face sets. Initially set them as type "patch" (not "wedge") — OpenFOAM will reject wedge type before the points are rotated:

FoamFile

{

version     2.0;

format      ascii;

class       dictionary;

object      createPatchDict;

}

pointSync false;

patches

(

{

name front;

patchInfo { type patch; }

constructFrom set;

set frontFaces;

}

{

name back;

patchInfo { type patch; }

constructFrom set;

set backFaces;

}

);

Run createPatch:

createPatch -overwrite

Warning: Do NOT set type "wedge" in createPatchDict. The faces must be rotated first. Setting wedge on flat faces causes a fatal "not planar" error.

## 5.2 Rotate Points to Wedge Geometry

Use the Python script rotatePoints.py to rotate all mesh points by ±2.5° around the X-axis. The script reads constant/polyMesh/points and transforms each point (x, y, z) as follows: if z > 0, new point = (x, y*cos(2.5°), y*sin(2.5°)); if z < 0, new point = (x, y*cos(2.5°), -y*sin(2.5°)).

python3 rotatePoints.py constant/polyMesh/points

The script creates a backup of the original points file as points.bak.

### Fix Degenerate Axis Cells

Cells along the symmetry axis (r = 0) collapse to zero volume in the wedge geometry, causing particle tracking errors. Use the fixAxisPoints2.py script to offset axis points slightly (1 micron) before re-applying the rotation. This script reads from the pre-rotation backup (points.bak):

python3 fixAxisPoints2.py constant/polyMesh/points

Warning: The fixAxisPoints2.py script must read from the original pre-rotation backup (points.bak). It offsets axis points in the flat slab, then re-applies the wedge rotation to all points. Running the rotation twice without starting from the backup will produce incorrect geometry.

## 5.3 Update Boundary Types

Edit constant/polyMesh/boundary and set the following patch types:

- front — type wedge

- back — type wedge

- axis — type symmetry

- inlet — type patch

- outlet — type patch

- wall — type wall

Also update the patch count at the top of the boundary file to match the total number of patches (6 in this case).

## 5.4 Verify the Wedge Mesh

checkMesh

Look for: no "Wedge patch is not planar" warnings, valid mesh geometry, and the bounding box should show a small Z extent corresponding to the ±2.5° wedge angle. Minor "no base point" warnings for axis faces are normal and can be ignored.

# 6. Configuring dsmcFoam+ Case Files

## 6.1 constant/dsmcProperties

This file defines the gas species, collision model, and simulation parameters. Key entries:

- coordinateSystem dsmcAxisymmetric;  — Required for wedge meshes.

- nEquivalentParticles  — Number of real molecules per simulator particle. Tune to get ~20 particles per cell. Start with 1e12 and adjust.

- axisymmetricProperties  — Contains maxRadialWeightingFactor (typically 1000).

- BinaryCollisionModel VariableHardSphere;  — Standard VHS collision model.

- typeIdList (EqGas);  — List of species names. Must match everywhere.

- moleculeProperties  — Contains mass, diameter, omega (viscosity index), alpha (VSS parameter), rotationalDegreesOfFreedom, and vibrationalModes for each species.

## 6.2 system/boundariesDict

Defines the DSMC behavior at each mesh boundary. There are three sections:

- dsmcPatchBoundaries  — Must contain one entry for every non-empty, non-wedge, non-symmetry patch. Common models: dsmcDiffuseWallPatch (wall), dsmcDeletionPatch (vacuum outlet and inlet).

- dsmcCyclicBoundaries  — Usually empty.

- dsmcGeneralBoundaries  — Contains the freestream inflow model (dsmcFreeStreamInflowPatch) which specifies the inlet species, temperature, velocity, and number density.

Warning: The number of entries in dsmcPatchBoundaries must exactly match the number of non-empty, non-wedge, non-symmetry patches in constant/polyMesh/boundary. A mismatch causes a fatal error. Wedge and symmetry patches are handled internally and do not need DSMC boundary models.

## 6.3 system/dsmcInitialiseDict

Defines the initial particle distribution in the domain. For a plume expanding into vacuum, initialize with a low-density background:

- type dsmcMeshFill;  — Fills the entire mesh.

- numberDensities { EqGas 1e18; }  — Low background density representing near-vacuum.

- translationalTemperature 300;  — Background temperature.

- velocity (0 0 0);  — Stationary background.

Warning: If nEquivalentParticles is too large relative to the background density and cell volume, zero particles will be created and the simulation will do nothing. If it is too small relative to the inlet density, initialization will take forever. Start with a low background density (1e18) and nEquivalentParticles of 1e12, then tune.

## 6.4 system/fieldPropertiesDict

Defines which macroscopic fields to compute from the particle data. The dsmcVolFields model computes density, velocity, temperature, pressure, etc. Key settings: resetAtOutput (on) resets sampling at each write; resetAtOutputUntilTime sets when to stop resetting and start accumulating time-averaged statistics.

## 6.5 system/controlDict

Standard OpenFOAM time control. Key settings for DSMC:

- application dsmcFoam+;

- deltaT  — Must be smaller than the mean collision time. For typical plume conditions, start with 1e-7 and reduce if needed.

- endTime  — Run for several flow-through times (domain length / inlet velocity) to reach steady state.

## 6.6 Other Files

- system/fvSchemes  — All entries set to "default none" (DSMC is Lagrangian, no PDE discretization).

- system/fvSolution  — Empty solvers block.

- system/controllersDict  — Empty state and flux controller lists.

- constant/dynamicMeshDict  — Set dynamicFvMesh to staticFvMesh for initial runs. Enable dynamicRefineFvMesh for adaptive mesh refinement once the case is verified working.

# 7. Running dsmcFoam+

## 7.1 Initialize Particles

dsmcInitialise+

This creates the 0/ directory containing the initial Lagrangian particle cloud and field files. Check that 0/lagrangian/dsmcCloud/ exists and contains particle data.

## 7.2 Run the Solver

dsmcFoam+ > log.dsmcFoam 2>&1 &

Monitor progress:

tail -f log.dsmcFoam

Check for new time directories appearing:

ls -lt | head

## 7.3 Quick Test Run

For initial verification, use conservative settings: nEquivalentParticles = 1e12, deltaT = 1e-7, endTime = 1e-5, writeInterval = 1e-6, dynamicFvMesh = staticFvMesh. This runs in seconds and confirms the case is properly configured. Scale up once verified.

## 7.4 Production Run Settings

For publication-quality results: nEquivalentParticles = 1e11 (or lower), deltaT = 3e-8 (less than mean collision time), endTime = 1e-3 (4+ flow-throughs), writeInterval = 1e-4. Enable adaptive mesh refinement if needed. These runs take hours and are best run on a cluster.

# 8. Post-Processing and Visualization

## 8.1 Opening in ParaView

- Create an empty case.foam file in the case directory if one does not exist: touch case.foam

- Open ParaView and load the case.foam file.

- Select the fields you want to view (rhoN_EqGas, Ttra_EqGas, Ma_EqGas, U_EqGas).

- Click Apply.

## 8.2 Recommended Visualization Settings

- rhoN_EqGas (number density) — Use log scale. This is the most visually informative field, showing the plume expansion cone. Apply Filters → Cell Data to Point Data for smoother rendering.

- Ttra_EqGas (translational temperature) — Shows cooling during expansion.

- Ma_EqGas (Mach number) — Shows supersonic expansion structure.

- Color maps: Use "Cool to Warm" or "Viridis" presets instead of the default rainbow.

## 8.3 Creating Animations

- File → Save Animation.

- Save as PNG image sequence (more reliable than AVI).

- Set resolution to 1920x1080.

- Stitch into video using ffmpeg:

ffmpeg -framerate 10 -i frame.%04d.png -c:v libx264 -pix_fmt yuv420p plume.mp4

# 9. Running on the UVM VACC Cluster

## 9.1 Transfer Case Files

scp -r myPlumeCase/ netid@vacc-user2.uvm.edu:~/OpenFOAM_Runs/

## 9.2 Transfer the Container Image

If using a custom Apptainer/Singularity image (.sif file), transfer it to the cluster:

scp dsmc_v3.sif netid@vacc-user2.uvm.edu:~/

## 9.3 Create a SLURM Submission Script

Create a file called run_dsmc.sh with SLURM directives specifying partition (general), time limit (24 hours for production runs), memory (32G), and the Apptainer exec commands to run dsmcInitialise+ followed by dsmcFoam+. Use the same environment setup pattern (module load apptainer, then apptainer exec with --bind flag) as tested with other cases.

## 9.4 Submit and Monitor

sbatch run_dsmc.sh

squeue -u netid

tail -f log.dsmcFoam

## 9.5 Retrieve Results

Once the job completes, download the results to your local machine for ParaView visualization, or use the VACC Open OnDemand web interface (https://ondemand.vacc.uvm.edu) to launch an interactive desktop session with ParaView.

# Appendix A: Complete File Listing

The following scripts are provided alongside this guide:

- topoSetDict — Splits frontAndBackPlanes into front/back face sets by normal direction.

- createPatchDict — Creates front and back patches from the face sets.

- rotatePoints.py — Rotates flat slab mesh points into ±2.5° wedge geometry.

- fixAxisPoints2.py — Offsets degenerate axis points (r ≈ 0) before re-applying wedge rotation.

- splitInlet.py — Splits the inlet patch into per-face patches and generates boundariesDict with spatially varying conditions interpolated from a CFD CSV file.

- run_dsmc_vacc.sh — SLURM submission script for the UVM VACC cluster.

# Appendix B: Troubleshooting

## Wedge patch is not planar

This occurs if wedge type is assigned before rotating the points, or if axis points were not properly handled. Solution: revert to the pre-rotation backup (points.bak), run fixAxisPoints2.py, then set patch types. Ensure the rotation is only applied once.

## Number of poly-patches not equal to patch models

dsmcFoam+ counts non-empty, non-wedge, non-symmetry patches and requires exactly that many entries in dsmcPatchBoundaries. Run "grep type constant/polyMesh/boundary" to see all patch types and count accordingly. Wedge and symmetry patches are excluded from the count.

## Cannot find patchField entry for [patchName]

The 0/ directory contains field files with old patch names. Delete the 0/ directory entirely and re-run dsmcInitialise+ to regenerate fields matching the current mesh patches.

## Segmentation fault during particle tracking

Usually caused by degenerate cells at the axis or by a patch without a DSMC boundary handler. Fix: ensure axis cells are offset with fixAxisPoints2.py, and verify that every non-empty/non-wedge/non-symmetry patch has an entry in dsmcPatchBoundaries.

## Simulation runs but no particles are created

The background number density (in dsmcInitialiseDict) multiplied by the cell volume, divided by nEquivalentParticles, yields the number of simulator particles per cell. If this is less than 1, no particles are inserted. Either decrease nEquivalentParticles or increase the initialization density.

## Simulation stuck on "Starting time loop"

dsmcFoam+ produces minimal terminal output during the time loop. Check that the process is using CPU (run "top" or "ps aux | grep dsmcFoam"). Check if time directories are being written. If the process exited silently, check the log file for errors. Most commonly this indicates zero particles were initialized.